# 1. Interface

The SPI module consists of 1 SPI Master and up to 8 SPI Slaves, connected via SCLK, MOSI, MISO, and SS lines.

Table 1: SPI Master - Signal Description

| Signal | Width | In/Out | Description |
| --- | --- | --- | --- |
| ref_clk | 1 | In | System reference clock |
| rst_n | 1 | In | Active-low asynchronous reset |
| in[7:0] | 8 | In | Data input (TX data or slave index) |
| out[7:0] | 8 | Out | Received data from slave |
| cntl[1:0] | 2 | In | Control opcode |
| ready | 1 | Out | High when master is in IDLE state |
| s_clk | 1 | Out | SPI clock (~ref_clk during TRANSFER, else 0) |
| mosi | 1 | Out | Master Out Slave In (= tx_reg[7]) |
| miso | 1 | In | Master In Slave Out |
| ss[7:0] | 8 | Out | Slave select, one-cold (active during LOAD_SS and TRANSFER) |

Table 2: SPI Slave - Signal Description

| Signal | Width | In/Out | Description |
| --- | --- | --- | --- |
| sclk | 1 | In | SPI clock from master |
| mosi | 1 | In | Data from master |
| miso | 1 | Out | Data to master |
| cs | 1 | In | Active-low chip select (from master ss) |
| clk | 1 | In | System clock for TX preload |
| rst_n | 1 | In | Active-low asynchronous reset |
| in[7:0] | 8 | In | TX data to preload |
| out[7:0] | 8 | Out | Received data from master |
| load | 1 | In | Latch in -> tx_load_reg (when cs=1) |
| ready | 1 | Out | High when slave is idle (= cs) |

# 2. Functional Implementation

SPI Communication module characteristics:

Data width: 8 bits per transaction, MSB-first.

Clock polarity: s_clk idle LOW, inverted ref_clk during transfer.

Sampling: slave samples MOSI on posedge s_clk (= negedge ref_clk). Master samples MISO on negedge ref_clk (= posedge s_clk).

Full-duplex: master sends and receives simultaneously in 8 clock cycles.

Slave select: one-cold encoding, supports up to 8 slaves.

Reset: asynchronous assert, active-LOW (rst_n).

Table 3: SPI Master CNTL Opcode

| CNTL | Action |
| --- | --- |
| 2'b00 | No operation |
| 2'b01 | Load in -> tx_reg (IDLE only) |
| 2'b10 | Load in -> slave_idx (IDLE only). If idx >= 8, ss stays 0xFF |
| 2'b11 | Start transfer. Not ready until done AND cntl != 2'b11 |

SPI Slave Operation:

load=1 and cs=1 (idle): latch in -> tx_load_reg.

cs asserts (1->0): transaction begins. Slave drives miso, samples mosi on sclk edges.

cs deasserts (0->1): transaction ends. out holds received byte.

Slave must have tx_load_reg valid before master asserts cs.

# 3. Internal Implementation

## 3.1. SPI Master Architecture

Registers:

tx_reg[7:0]: TX shift register. Shift left, fill 0 each posedge ref_clk during TRANSFER.

rx_reg[7:0]: RX shift register. Sample miso each negedge ref_clk during TRANSFER.

bit_cnt[2:0]: Bit counter 0-7 during TRANSFER, reset otherwise.

slave_idx[7:0]: Binary slave index. Load via cntl=10. Reset = 0xFF (no slave).

Combinational outputs:

mosi = tx_reg[7]

out = rx_reg

ss = ~(1 << slave_idx[2:0]) when (LOAD_SS or TRANSFER) and slave_idx < 8, else 0xFF

s_clk = ~ref_clk when TRANSFER, else 0

ready = (state == IDLE)

## 3.2. SPI Master State Machine

IDLE -> LOAD_SS -> TRANSFER -> WAIT_CLEAR -> IDLE

| State | Transition condition | Description |
| --- | --- | --- |
| IDLE | cntl == 2'b11 | Accept load commands, ready=1 |
| LOAD_SS | Unconditional (1 cycle) | Assert ss one cycle before s_clk starts |
| TRANSFER | bit_cnt == 7 | Shift 8 bits, s_clk = ~ref_clk |
| WAIT_CLEAR | cntl == 2'b00 | Hold until host releases cntl, prevents retrigger |

## 3.3. SPI Slave Architecture

Registers:

tx_load_reg[7:0]: Holding register. Latched on posedge clk when cs=1 and load=1.

tx_shift_reg[7:0]: TX shift register. Clocked by negedge sclk. Reloaded on posedge cs (deassert).

rx_reg[7:0]: RX shift register. Clocked by posedge sclk when cs=0.

first_bit: 1-bit flag. Set on posedge cs, cleared on first negedge sclk after cs asserts.

MISO output mux:

cs=1 (idle): miso = 0

cs=0, first_bit=1: miso = tx_load_reg[7] (look-ahead for first bit)

cs=0, first_bit=0: miso = tx_shift_reg[7]

# 4. History

| Date | Author | Modified part | Description |
| --- | --- | --- | --- |
| 2026/05/20 | Nguyen To Quoc Viet | All | New creation |